List of Objectives and Constraints

Our objective is to create a battery pack with the energy required to finish the race

- In our first stages we are using Power to come up with a preliminary S&P configuration

- We don’t know how much energy is required

- It is the Data Generation Teams job to provide us with the voltage and current required to complete the race

  - Once they give us those requirements we will verify that our inverters and motors can handle that voltage and current

  - We will calculate energy required based off the voltage and current requirements from the Data Generation Team

- Our final S&P configuration depends the energy required

Constraints

- 400 V is our voltage cap due to Inverter operating limit

- 300 A is our current cap due to motor operating limit (150 for each motor)

- 80 kW is our power cap due to competition regulations (40 kW for each motor)

  - Peak singular motor power is 70 kW but can only be operated for a short burst of time.

  - The motors may need a cooling system in order to operate with higher power

    - For Continuous Motor Power: Air cooled -> 28 kW max/motor, Liquid cooled -> 30 kW max/motor, Combined air & liquid cooled -> 35 kW max/motor

    - We may not need a battery pack capable of 80 kW max power

Specs from last years car

- Pack Volume (just cells)= 2698 in^3

- # of cells = 432

- Configuration = 108s4p

- Power = 81.6 kW (not accounting inverter losses)

- Max TS Voltage = 453.6 V

- Nominal TS Voltage = 388.8 V

- Current = 180 A (60 A to each motor, they had 100 A fuses)

- Their motor (EMRAX 208) could operate at 40 kW continuous motor power with combined air & liquid cooling system